ELECTRONIC DIGITAL CALIPER

OPERATING INSTRUCTIONS

- APPLICATIONS:

1.Comparisons between plug and hole                             2.Deviation from reference size A

3.Measuring of bottom thickness    4.Measuring of centre distance      5.Measuring where display

between two identical holes        faces opposite

- INSTRUCTIONS

- Keep body face clean, prevent liquid material from getting into slider to destroy electronics

- Face should be cleared by petrol.,body lubricated by few clock oil.Acetone and alcohol mustn’t be used.

- Don’t remove end lid without using output intereface.Output end shouldn’t be contacted with metal in order to protect electronics against damage.

- Other notes refer to conventional caliper.

●SPECIFICATIONS AND FUNCTIONS:

Repeatability.              0.01mm or 0.0005”

Maximum measuring speed:   1.5m/sec or 60”/sec

Measuring system:           non contaction linear CAP measuring system.

Display:                   LCD display,minus sign “-”, character in 4.7mm/0.185” height for 5digits, small digit“5”

and “IN” sign for inch measuring unit.

Battery :                   one silver oxide battery 1.5V for one year of continuos use. Low of battery warning

by flashing display

Operating temperature:       0℃- +40℃

Storage temperature:         -20℃-+70℃.

Influence of humidity:        not important within 0 to 80% of relatieve humifity

Data output:                serial output for interface with host computer or printer

Data processing

Interface (option):             Functions: data storage ,processing & printing.with software of quality control.

- TROUBLE SHOOTING

| Trouble | Cause | Remedy |
| --- | --- | --- |
| Five digits jump simultaneously once per second about | Battery voltage is under 1.45V | Remove the cover and replace battery |
| The display can’t count. | Circuit accident | Put out battery and reset it in after 30sec. |
| Only display 000.00 or IN 00.00 | Both function buttons and slider’s signal end short out. | Remove the cover, put button’s spring in idea condition |
| Function buttons non active | Spring out of shape caused by violent pressing buttons | Ditto |
| Reading error for full length is ≤0.1mm | Dirts get into sensor . | Remove the cover and slider assembly ,jet sensor’s face with clean pressure air (≤51kg/cm²), clear it with fewpetrol for dirt preventing |
| LCD screen no display | - Battey hasn’t good contact. - Battey voltage is under 1.1V | - Remove the cover.keep good contact for battery - Replace battery |

  - METRIC/INCH change over (true conversion)

  - Zero setting (zero setting at any position within the measuring  range)

  - Battery change cover

  - Data output socket

Battery replacement